# Task Report: Week 1

Network Maintenance and Monitoring Setup

| Domain | Network Maintenance and Monitoring Setup |
| --- | --- |
| Simulation platform | Cisco Packet Tracer |
| Report type | Practical Implementation & Performance Verification Report |

## 1. Executive Summary & Objective

This project designs, deploys, and verifies a local area network (LAN) environment with integrated performance monitoring and service delivery. The primary objectives achieved include:

- Designing a star network topology linking a central Cisco 2911 router, a Cisco Catalyst 2960 switch, local server infrastructure, and client workstations.

- Implementing a static IPv4 addressing scheme alongside local DNS domain resolution and HTTP service hosting.

- Validating active diagnostic mechanisms using ICMP echo requests (ping), route tracing (tracert), and an SNMP MIB Browser manager to inspect router system parameters.

- Documenting operational observations, root-cause troubleshooting steps, and network maintenance recommendations.

## 2. Network Topology & Addressing Schema

The network uses a centralized star topology. Host workstations and server infrastructure connect directly into a central Cisco Catalyst 2960-24TT Switch (Switch0), which is connected via an uplink interface to a Cisco 2911 Router (Router0) serving as the Default Gateway.     Figure 1: Network Logical Topology Layout (task1.pkt)

Addressing Matrix

| Device | Interface | IPv4 address | Subnet mask | Default gateway | Primary service / role |
| --- | --- | --- | --- | --- | --- |
| Router0 | Gig0/0 | 10.10.10.1 | 255.0.0.0 | N/A | Default Gateway |
| Server0 | FastEth0 | 10.10.10.5 | 255.0.0.0 | 10.10.10.1 | DNS Server & HTTP Web Host (university.local) |
| PC0 | FastEth0 | 10.10.10.2 | 255.0.0.0 | 10.10.10.1 | Diagnostic & Management Workstation |
| PC1 | FastEth0 | 10.10.10.3 | 255.0.0.0 | 10.10.10.1 | LAN End-Node Workstation |
| PC2 | FastEth0 | 10.10.10.4 | 255.0.0.0 | 10.10.10.1 | LAN End-Node Workstation |

Figure 1: Network Logical Topology Layout (task1.pkt)

# 3. Configuration Scripts & Parameters

### A. Router Gateway & SNMP Agent Setup (Router0)

Cisco CLI

Router> enable

Router# configure terminal

Router(config)# hostname Router0

Router0(config)# interface GigabitEthernet0/0

Router0(config-if)# ip address 10.10.10.1 255.0.0.0

Router0(config-if)# no shutdown

Router0(config-if)# exit

! SNMP Agent Setup for Network Monitoring

Router0(config)# snmp-server community public RO

Router0(config)# snmp-server location Lab-Floor-1

Router0(config)# snmp-server contact Admin

Router0(config)# end

Router0# write memory

### B. Host Network Configuration (PC0)

- IP Configuration Mode: Static

- IPv4 Address: 10.10.10.2

- Subnet Mask: 255.0.0.0

- Default Gateway: 10.10.10.1

- DNS Server: 10.10.10.5

Figure 2: Host Interface IP Configuration (PC0)

### C. Server Application Services (Server0)

- DNS Service Setup:

  - Status: Enabled

  - Record Name: university.local

  - Record Type: A Record

  - Target Resolution: 10.10.10.5

- HTTP Service Setup:

  - Status: Enabled

  - Content Hosted: Default web portal interface (index.html) served over port 80.

## 4. Network Monitoring & Performance Verification

### A. Gateway Reachability & Hop Trace (PC0 → Router0)

An ICMP polling test and route tracing operation were executed from PC0 to verify gateway pathing and latency metrics:

Plaintext

C:\>ping 10.10.10.1

Pinging 10.10.10.1 with 32 bytes of data:

Reply from 10.10.10.1: bytes=32 time=199ms TTL=255

Reply from 10.10.10.1: bytes=32 time=1ms TTL=255

Reply from 10.10.10.1: bytes=32 time<1ms TTL=255

Reply from 10.10.10.1: bytes=32 time<1ms TTL=255

Ping statistics for 10.10.10.1:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 199ms, Average = 50ms

C:\>tracert 10.10.10.1

Tracing route to 10.10.10.1 over a maximum of 30 hops:

1   0 ms      0 ms      0 ms      10.10.10.1

Trace complete.

- Packet Delivery: 100% success rate (0% packet loss).

- Transit Verification: Route trace confirmed a direct 1-hop path with 0ms internal processing latency.

Figure 3: Gateway ICMP Ping and Trace Diagnostics

### B. Inter-Node Workstation Verification (PC0 → PC1)

Local layer-2 communication was verified across switch access ports by pinging workstation PC1 (10.10.10.3):

Plaintext

C:\>ping 10.10.10.03

Pinging 10.10.10.03 with 32 bytes of data:

Reply from 10.10.10.3: bytes=32 time<1ms TTL=128

Reply from 10.10.10.3: bytes=32 time<1ms TTL=128

Reply from 10.10.10.3: bytes=32 time<1ms TTL=128

Reply from 10.10.10.3: bytes=32 time<1ms TTL=128

Ping statistics for 10.10.10.3:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 0ms, Maximum = 0ms, Average = 0ms

- Observed Performance: Sub-1ms latency across all packets with zero packet drop.

Figure 4: Workstation-to-Workstation Subnet Ping Diagnostics

### C. Domain Name System (DNS) & HTTP Service Verification

Web application availability was evaluated from PC0's browser:

- Target URL: [http://university.local](http://university.local)

- Verification Flow: PC0 queries DNS Server 10.10.10.5 → Name resolves to target IP → HTTP GET request sent over TCP port 80 → Web page successfully displayed.

Figure 5: DNS Name Resolution and Web Server Delivery

### D. SNMP Management & MIB Traversal

Network device monitoring was performed using the built-in MIB Browser management tool on PC0:

- Target Managed Address: 10.10.10.1

- SNMP Operation: Get

- Target MIB Branch: router_std MIBs → .iso.org.dod.internet.mgmt.mib-2.system.sysName

- Queried OID: .1.3.6.1.2.1.1.5.0

- Result Value: Router (Type: OctetString)

The SNMP response confirms that the monitoring station successfully communicates with the router's SNMP agent and retrieves device status metrics.

Figure 6: SNMP MIB Browser Query Result (sysName)

## 5. Troubleshooting Analysis Log

| Issue ID | Identified Symptom | Root Cause | Corrective Action & Verification |
| --- | --- | --- | --- |
| TS-01 | First ICMP packet to 10.10.10.1 experienced a 199ms delay. | Initial ARP table miss; the switch needed to flood an ARP broadcast request to discover the gateway MAC address. | Normal behavior during initial packet transmission; subsequent ICMP iterations maintained stable latency under 1ms. |
| TS-02 | Web browser error when requesting [http://university.local](http://university.local). | Missing DNS Server IP in PC0's configuration; the host had no valid resolver for local hostnames. | Configured DNS Server field to 10.10.10.5 in PC0 Global Settings. Re-opened browser and confirmed page loaded instantly. |
| TS-03 | SNMP manager query timeouts during early testing. | Mismatch in community string parameters and improper write privileges on the router agent. | Standardized the router configuration to Read-Only access using community string public, cleared Write string fields in the manager, and confirmed data retrieval under .1.3.6.1.2.1.1.5.0. |

## 6. Recommendations & Network Maintenance Plan

- VLAN Segmentation: Partition workstation nodes and server assets into isolated Virtual LANs (e.g., VLAN 10 for Workstations, VLAN 20 for Servers) via router-on-a-stick sub-interfaces to reduce broadcast traffic.

- Centralized DHCP Provisioning: Configure a DHCP IP pool on Router0 with pre-defined DNS and gateway options to eliminate manual static IP allocation errors.

- SNMP Protocol Hardening: Upgrade from SNMPv2c plaintext community strings to SNMPv3 to ensure authenticated SHA hashing and DES/AES packet encryption.